CMPE314: Principles of Electronic Circuits

Dr. Yan

Lab 03 Report:

Clippers and Clampers

Sabbir Ahmed

Yu Fu

- Objective

Construct and measure clipper and clamper circuits and study their characteristics.

- Equipment

  - Resistors; 2 × 10 kΩ, 1 × 100 kΩ

  - Capacitors; 1 × 0.22 µF

  - Diodes; 2 × 1N4738, 1 × 1n4740

  - Oscilloscope, DC power supply, digital multi-meter, function generator, breadboard

- Background

Clipper circuits limit or constrain signals by clipping off part of the signal in some region or compressing it in that region using a resistive voltage divider. Clamper circuits set a minimum and maximum value on an output AC waveform by shifting the DC level of the steady-state AC signal instead of altering its shape.

- Procedures

4.1     Part A. Clipper Circuit I

Figure 1: Clipper Circuit Without Resistor

  - Use a 1N4738 diode to construct the circuit from Figure 1.

  - Set the input signal amplitude to be 6 V and frequency to be 10 kHz.

  - Capture the input and output voltages on the oscilloscope with the input being a sinusoidal, square and saw-tooth waveform, respectively.

  - Save the sinusoidal input and output waveform data to plot alongside their theoretical computation.

4.2     Part B. Clipper Circuit II

Figure 2: Clipper Circuit with Resistor

- Repeat Part A with Figure 2.

4.3    Part C. Clamper Circuit

Figure 3: Clamper Circuit with VB = 5 V

- Use a 1N4740 diode to construct the circuit from Figure 3.

- Set the input signal amplitude to be 4 V and frequency to be 10 kHz.

- Capture the input and output voltages on the oscilloscope with the input being a sinusoidal and a square wave with +2 V peaks and -4 V valleys, respectively.

- Save the sinusoidal input and output waveform data to plot alongside their theoretical computation.

Figure 4: Clamper Circuit with VB = 1 V

- Repeat steps a. to d. with VB = 1 V as in Figure 4.

- Results

The clipper circuit from Figure 1 was constructed and the waveform on the oscilloscope was captured.

Figure 5: Waveform of Figure 1 on the Oscilloscope with Sinusoidal Inputs

The plot data was dumped and simulated as shown below in Figure 6.

Figure 6: Simulation of the Figure 1 Waveform

The waveform was computed theoretically and simulated as well. The input sinusoidal was Vamp×sin2πf→6sin2π×10kHz→6sin20000π. The output waveform was constructed as a piecewise function, where the output was clipped at its peak when it reached VB1+Vγ=2.7 V and at its valley when VO=-VB2+Vγ=-4.7 V. The following output was constructed.

Figure 7: Simulation of the Theoretical Waveforms of Figure 1

Figure 8: Simulation of Both the Dumped and Theoretically Computed Waveforms of Figure 1

The procedure was repeated with a square wave and a saw tooth input and captured on the oscilloscope.

Figure 9: Waveforms of Figure 1 on the Oscilloscope with Square and Saw-tooth Wave Inputs

Identical steps were taken to generate waveforms for the clipper in Figure 2.

Figure 10: Waveform of Figure 2 on the Oscilloscope with Sinusoidal Inputs

The plot data was dumped and simulated as shown below in Figure 11.

Figure 11: Simulation of the Figure 2 Waveform

The waveform was computed theoretically and simulated as well. The input sinusoidal was 6sin20000π. The output waveform was constructed as a piecewise function, where the output was clipped at its peak when it reached VB1+Vγ=2.7 V and at its valley when VO=-VB2+Vγ=-4.7 V. The following output was constructed.

Figure 12: Simulation of the Theoretical Waveforms of Figure 2

Figure 13: Simulation of Both the Dumped and Theoretically Computed Waveforms of Figure 2

The procedure was repeated with a square wave and a saw tooth input and captured on the oscilloscope.

Figure 14: Waveforms of Figure 2 on the Oscilloscope with Square and Saw-tooth Wave Inputs

Identical steps were taken to generate waveforms for the clamper circuit in Figure 3.

Figure 15: Waveform of Figure 3 on the Oscilloscope with Sinusoidal Inputs

The plot data was dumped and simulated as shown below in Figure 16.

Figure 16: Simulation of the Figure 3 Waveform

The waveform was computed theoretically and simulated as well. The input sinusoidal was 4sin20000π. The output waveform was simply the input function shifted up VB+Vγ +manual offest=5.7+2=7.7 V. The following output was constructed.

Figure 17: Simulation of the Theoretical Waveforms of Figure 3

Figure 18: Simulation of Both the Dumped and Theoretically Computed Waveforms of Figure 3

The procedure was repeated with a square wave input with an offset of +2 V and captured on the oscilloscope.

Figure 19: Waveforms of Figure 3 on the Oscilloscope with Square Wave Inputs

Identical steps were taken to generate waveforms for the clamper circuit in Figure 4.

Figure 20: Waveform of Figure 4 on the Oscilloscope with Sinusoidal Inputs

The plot data was dumped and simulated as shown below in Figure 21.

Figure 21: Simulation of the Figure 3 Waveform

The waveform was computed theoretically and simulated as well. The input sinusoidal was sin20000π. The output waveform was simply the input function shifted up VB+Vγ +manual offest=1.7+2.8=4.5 V. The following output was constructed.

Figure 22: Simulation of the Theoretical Waveforms of Figure 4

Figure 23: Simulation of Both the Dumped and Theoretically Computed Waveforms of Figure 3

The procedure was repeated with a square wave input with an offset of +2 V and captured on the oscilloscope.

Figure 24: Waveforms of Figure 4 on the Oscilloscope with Square Wave Inputs

- Conclusion

The characteristics of clipper and clamper circuits, although appear simplistic, have many uses in real world applications. Simulating the waveforms mathematically was possible without much difficulty as demonstrated.